Keyboard Acoustic Side-Channel Attack

CNN + Mel Spectrogram Pipeline

A Complete Beginner's Guide

Based on: github.com/ayushma18/keyboard_sound ¦ CNN.ipynb Covers: Audio → Mel Spectrogram → CNN → Keystroke Classification

# Chapter 1: What Is This Project?

This project is a keyboard acoustic side-channel attack — a type of security research that asks a provocative question:

🔑 Core Question: Can we listen to the sound of someone typing and figure out which keys they pressed?

The answer is yes — and this notebook shows you exactly how. The approach uses machine learning (specifically a Convolutional Neural Network, or CNN) trained on audio recordings of individual keystrokes. Each key produces a slightly different sound. By learning to recognize those differences, the model can classify which key was pressed just from the audio.

## 1.1  Why This Matters

This has real-world security implications. An attacker could:

- Record audio of someone typing a password over a Zoom call

- Use a phone's microphone or a nearby IoT device

- Reconstruct text without ever seeing the screen

Understanding this attack helps defenders build better protections — acoustic noise generators, randomized typing delays, or keystroke-level privacy screens.

## 1.2  The Big Picture Pipeline

📋 Step-by-Step Overview: Raw Audio (.wav) → Load with Librosa → Slice to keystroke segments → Convert to Mel Spectrogram → Normalize → Feed into CNN → Predict which key was pressed

Each of those steps involves specific decisions about parameters. This guide explains every one.

# Chapter 2: Raw Audio — What You Start With

## 2.1  What Is a WAV File?

A WAV file stores audio as a list of numbers called samples. Each sample measures the amplitude (loudness/pressure) of the sound wave at a specific moment in time.

🎵 Analogy: Think of sound as ripples in water. A WAV file is like writing down the height of the water surface 44,100 times per second.

## 2.2  Key Concepts Before We Touch Any Code

### Sample Rate (sr)

Sample Rate is how many audio samples are recorded per second. It is measured in Hz (Hertz).

# Common sample rates:

sr = 22050    # Default for librosa — 22,050 samples per second

sr = 44100    # CD quality — 44,100 samples per second

sr = 16000    # Speech recognition standard

If sr = 22050, then a 1-second audio clip has exactly 22,050 numbers. A 0.1-second clip has 2,205 numbers.

⚠️ Why does this matter? Higher sample rate = more detail = larger file = more computation. For keystroke sounds (short, percussive), 22050 Hz is usually enough. Some researchers use 44100 to capture higher-frequency content.

### Amplitude

Each sample value represents the amplitude of the sound wave — how loud it is at that moment. Librosa typically normalizes audio to float values between -1.0 and +1.0.

### Duration

For keystroke detection, each audio clip is usually very short — around 0.05 to 0.2 seconds. The project isolates individual key presses so the model only sees one keystroke at a time.

## 2.3  Loading Audio with Librosa

import librosa

import numpy as np

# Load audio file

audio, sr = librosa.load('key_a_press.wav', sr=22050)

# audio → numpy array of shape (n_samples,)

# sr    → sample rate = 22050

print(f'Duration: {len(audio)/sr:.3f} seconds')

print(f'Total samples: {len(audio)}')

print(f'Audio range: [{audio.min():.3f}, {audio.max():.3f}]')

📌 librosa.load() default behavior: If you don't specify sr, librosa resamples everything to 22050 Hz automatically. This ensures all your files have the same sample rate even if they were recorded differently.

# Chapter 3: The Mel Spectrogram — The Heart of the Pipeline

This is the most important chapter. Everything before this is setup; everything after this is machine learning. The mel spectrogram is the bridge between raw audio and the CNN.

## 3.1  Why Can't We Just Feed Raw Audio to the CNN?

You could — but it would be inefficient. A 0.1-second clip at 22050 Hz has 2,205 numbers. A CNN treating these as a 1D signal struggles to find patterns because:

- Adjacent samples differ by microseconds — hard for the model to learn structure

- The signal has no explicit frequency information

- CNNs excel at 2D image-like data, not 1D time series

The solution: convert audio into a 2D image-like representation that captures both time and frequency simultaneously. That's the spectrogram.

## 3.2  Step 1 — The Fourier Transform (Understanding Frequency)

Before we can make a spectrogram, you need to understand what a Fourier Transform does.

🔬 Fourier Transform Intuition: Any complex sound is actually a mixture of pure tones (sine waves) at different frequencies. A Fourier Transform decomposes the sound into its component frequencies — telling you 'how much of each frequency is present.'

Example: The sound of the letter 'A' key being struck contains:

- A sharp click (broad-spectrum, high-frequency content)

- The resonance of the key mechanism (low-mid frequency)

- The thud of finger contact (very low frequency)

Different keys have different proportions of these frequencies, which is exactly what makes them distinguishable.

## 3.3  Step 2 — The Short-Time Fourier Transform (STFT)

A regular Fourier Transform analyzes the entire clip at once — you lose track of when each frequency occurred. The STFT fixes this by analyzing the audio in small overlapping windows.

# STFT is computed internally when making a mel spectrogram

# But here's what it does conceptually:

# 1. Take a short window of audio (e.g., 23ms at 22050 Hz = 512 samples)

# 2. Run FFT on that window

# 3. Slide the window forward by hop_length samples

# 4. Repeat until you've covered the entire signal

# Result: a 2D matrix [frequencies × time_frames]

### Key STFT Parameters

| Parameter | Typical Value | Range | What It Controls |
| --- | --- | --- | --- |
| n_fft | 2048 | 512 – 8192 | Window size in samples. Determines frequency resolution. Larger = more frequency detail, less time detail. |
| hop_length | 512 | 128 – 1024 | How far to slide the window each step. Controls time resolution. Smaller = more frames, more detail, more memory. |
| win_length | n_fft | < n_fft | Actual analysis window. Often equal to n_fft. Smaller windows give better time resolution. |
| window | 'hann' | Various | Shape applied to each window to reduce edge artifacts. Hann window is standard. |

📐 Time-Frequency Tradeoff: Larger n_fft = better frequency resolution but worse time resolution. This is the Heisenberg Uncertainty Principle applied to signal processing — you can't maximize both simultaneously.

## 3.4  Step 3 — The Mel Scale (Making It Human-Perceptual)

The STFT gives you frequencies in Hz. The problem: humans don't perceive frequency linearly. We can easily distinguish 100 Hz from 200 Hz (a big difference), but we can barely distinguish 10,000 Hz from 10,100 Hz (same 100 Hz difference).

👂 The Mel Scale: The Mel scale maps Hz to a perceptual scale that matches how human ears actually work. Equal distances on the Mel scale sound equally different to human listeners.

The conversion formula is:

# Hz to Mel conversion:

mel = 2595 * log10(1 + hz / 700)

# Examples:

#  100 Hz → 150 Mel

#  500 Hz → 607 Mel

# 1000 Hz → 999 Mel

# 4000 Hz → 1740 Mel

# 8000 Hz → 2146 Mel

Instead of treating all frequencies equally, mel filterbanks group higher frequencies into broader bins — fewer bins cover the high-frequency range where human perception (and keystroke discrimination) is less sensitive.

## 3.5  Step 4 — Putting It All Together: librosa.feature.melspectrogram()

import librosa

import numpy as np

def audio_to_melspectrogram(audio, sr=22050):

mel_spec = librosa.feature.melspectrogram(

y=audio,          # Input audio signal (numpy array)

sr=sr,            # Sample rate

n_mels=128,       # Number of Mel frequency bins

n_fft=2048,       # FFT window size

hop_length=512,   # Hop length between windows

fmin=0,           # Minimum frequency (Hz)

fmax=sr/2,        # Maximum frequency (Nyquist limit)

)

return mel_spec

# Output shape: (n_mels, time_frames) = (128, T)

# Where T depends on audio length and hop_length

## 3.6  Every Parameter Explained in Depth

### Parameter: y (the audio signal)

This is your raw audio — a 1D numpy array of floats. Each value is an amplitude measurement. This is what librosa.load() returns.

audio, sr = librosa.load('key_press.wav', sr=22050)

# audio.shape → (n_samples,)  e.g., (2205,) for 0.1 seconds

### Parameter: sr (Sample Rate)

Typical value: 22050 Hz

The number of samples per second. Must match the actual sample rate of your audio. Librosa will resample if you specify a different sr than the file's original rate.

- Lower sr (e.g., 16000) → smaller spectrograms, faster training, less high-frequency detail

- Higher sr (e.g., 44100) → larger spectrograms, more detail, more compute

### Parameter: n_mels

Typical value: 128  |  Range: 40 – 256

The number of Mel frequency bins in the output spectrogram. Think of this as the height of your spectrogram image.

n_mels=40   # Smaller → coarser frequency resolution, faster

n_mels=128  # Standard → good balance for keystroke classification

n_mels=256  # Larger → finer frequency detail, more memory

Each of the n_mels bins represents a range of frequencies on the Mel scale. Bin 0 = lowest frequencies (near fmin), Bin 127 = highest frequencies (near fmax).

💡 For keystrokes: 128 mel bins is generally more than enough. Keyboard sounds are not musically complex. Some researchers use 64 bins and still achieve high accuracy.

### Parameter: n_fft

Typical value: 2048  |  Range: 512 – 8192

The size of the FFT window — the number of audio samples analyzed at once per frame. This directly controls frequency resolution.

# At sr=22050:

n_fft=512   → window = 512/22050 = 23ms   # Short, less frequency detail

n_fft=1024  → window = 1024/22050 = 46ms  # Medium

n_fft=2048  → window = 2048/22050 = 93ms  # Standard, good frequency detail

n_fft=4096  → window = 4096/22050 = 186ms # Long, very fine frequencies

The frequency resolution of each FFT frame is: Δf = sr / n_fft

# Frequency resolution examples:

# sr=22050, n_fft=2048 → Δf = 22050/2048 ≈ 10.8 Hz per bin

# sr=22050, n_fft=512  → Δf = 22050/512  ≈ 43.1 Hz per bin

# So n_fft=2048 can distinguish frequencies 10.8 Hz apart

# n_fft=512 can only distinguish frequencies 43 Hz apart

⚠️ Critical Tradeoff: Larger n_fft = better frequency resolution BUT the window covers more time, so you blur the time information. For fast events like keystrokes (10–50ms), a smaller n_fft may actually be better.

### Parameter: hop_length

Typical value: 512  |  Range: 128 – 1024

After each FFT analysis window, hop_length is how many samples we move forward before computing the next frame. This controls the time resolution of the spectrogram.

# Time resolution = hop_length / sr

hop_length=512  → time step = 512/22050 = 23.2ms per frame

hop_length=256  → time step = 256/22050 = 11.6ms per frame

hop_length=128  → time step = 128/22050 = 5.8ms per frame

# Number of frames from a 0.1 second clip:

# hop=512:  n_frames = floor(2205/512) + 1 = 5 frames

# hop=256:  n_frames = floor(2205/256) + 1 = 9 frames

# hop=128:  n_frames = floor(2205/128) + 1 = 18 frames

Smaller hop_length = more time frames = wider spectrogram image = more data = more computation. The overlap between consecutive windows helps smooth the analysis.

### Parameter: fmin and fmax

Typical values: fmin=0 Hz, fmax=sr/2 (Nyquist)

These define the frequency range that your mel filterbank covers.

# Nyquist theorem: max detectable frequency = sr/2

fmax = sr / 2 = 22050 / 2 = 11025 Hz  # Maximum for sr=22050

# Narrow range example (focus on relevant frequencies):

librosa.feature.melspectrogram(y=audio, sr=sr, fmin=50, fmax=8000)

# Why limit fmax?

# Keyboard sounds mostly live below 8kHz

# Limiting fmax focuses your n_mels bins on the relevant range

💡 Practical tip: If you use n_mels=128 and fmax=11025, those 128 bins span 0–11025 Hz. If you set fmax=4000, the same 128 bins now zoom into a narrower band, giving finer resolution exactly where keystroke energy lives.

## 3.7  Converting Power to Decibels (Log Compression)

The raw mel spectrogram values are in power (energy squared). These values span several orders of magnitude — the loudest frequency bin might be 1,000,000x stronger than the quietest. CNNs struggle with such extreme value ranges.

The solution: convert to decibels (dB), which uses a logarithmic scale.

# Convert power spectrogram to dB scale

mel_spec = librosa.feature.melspectrogram(y=audio, sr=sr, n_mels=128)

mel_spec_db = librosa.power_to_db(mel_spec, ref=np.max)

# ref=np.max normalizes so the loudest value becomes 0 dB

# All other values are negative (e.g., -80 dB = very quiet)

# Result: values roughly in range [-80, 0]

# Much friendlier for neural networks than [0, 1000000]

| Parameter | Typical Value | Range | What It Controls |
| --- | --- | --- | --- |
| ref=np.max | Dynamic | Any function | The reference point for 0 dB. np.max makes the loudest value 0 dB and everything else negative. |
| ref=1.0 | Fixed | Any float | Fixed reference. Values can be positive or negative. Less common. |
| top_db | 80.0 | 40 – 120 | Clips values below (0 - top_db) dB. Removes near-silence. Default is 80 dB dynamic range. |

## 3.8  Visualizing the Mel Spectrogram

import matplotlib.pyplot as plt

import librosa.display

fig, ax = plt.subplots(figsize=(10, 4))

img = librosa.display.specshow(

mel_spec_db,

x_axis='time',       # X-axis shows time

y_axis='mel',        # Y-axis shows Mel frequencies

sr=sr,

hop_length=hop_length,

ax=ax

)

plt.colorbar(img, ax=ax, format='%+2.0f dB')

plt.title('Mel Spectrogram of Keystroke')

# What you'll see:

# - X axis: time (left = start, right = end of keystroke)

# - Y axis: frequency (bottom = low, top = high)

# - Color: intensity (bright = loud at that freq/time, dark = quiet)

# Chapter 4: The Data Pipeline

## 4.1  Data Collection

Before any machine learning can happen, you need labeled audio samples. In this project, data collection typically works like this:

- Record yourself pressing each key multiple times while audio is captured

- Detect the moment of each keypress (using amplitude thresholding)

- Slice the recording into individual keystroke segments

- Label each segment with the correct key

- Save as individual .wav files organized by class (key)

# Typical data directory structure:

Data/

key_a/

press_001.wav

press_002.wav

...

key_s/

press_001.wav

...

key_d/

...

## 4.2  Keystroke Detection / Segmentation

Raw recordings contain silence between keystrokes. The data pipeline must detect where each keypress occurs.

def detect_keystrokes(audio, sr, threshold=0.02, pad_ms=50):

'''

Detect individual keystrokes in a recording.

threshold: amplitude above which we consider a keystroke started

pad_ms: milliseconds of audio to include before/after the event

'''

pad_samples = int(pad_ms * sr / 1000)

# Find samples above threshold

above_threshold = np.abs(audio) > threshold

# Find rising edges (start of each keystroke)

# ... segment extraction logic ...

return segments

## 4.3  Fixed-Length Padding

CNNs require all input images to be the same size. Since different keystrokes may have slightly different durations, we pad or truncate all segments to a fixed length.

TARGET_LENGTH = 0.1  # seconds

TARGET_SAMPLES = int(TARGET_LENGTH * sr)  # = 2205 at sr=22050

def pad_or_truncate(audio, target_length):

if len(audio) >= target_length:

return audio[:target_length]      # Truncate if too long

else:

pad = target_length - len(audio)

return np.pad(audio, (0, pad))    # Zero-pad if too short

## 4.4  Full Feature Extraction Pipeline

def extract_features(audio_path, sr=22050, n_mels=128,

n_fft=2048, hop_length=512):

'''

Full pipeline: audio file → mel spectrogram ready for CNN

'''

# Step 1: Load audio

audio, sr = librosa.load(audio_path, sr=sr)

# Step 2: Pad/truncate to fixed length

audio = pad_or_truncate(audio, int(0.1 * sr))

# Step 3: Compute mel spectrogram

mel = librosa.feature.melspectrogram(

y=audio, sr=sr,

n_mels=n_mels,

n_fft=n_fft,

hop_length=hop_length

)

# Step 4: Convert to dB

mel_db = librosa.power_to_db(mel, ref=np.max)

# Step 5: Add channel dimension for CNN

# Shape: (n_mels, time_frames) → (n_mels, time_frames, 1)

mel_db = mel_db[..., np.newaxis]

return mel_db

📐 Shape at each step: audio (2205,) → mel (128, 5) → mel_db (128, 5) → mel_db with channel (128, 5, 1). This final shape is what the CNN receives.

## 4.5  Normalization

Before feeding data into the CNN, we normalize the spectrogram values so they have a consistent scale across all samples.

# Method 1: Min-Max Normalization → scale to [0, 1]

mel_normalized = (mel_db - mel_db.min()) / (mel_db.max() - mel_db.min())

# Method 2: Z-score Normalization → mean=0, std=1

mel_normalized = (mel_db - mel_db.mean()) / mel_db.std()

# Method 3: Global statistics (computed across all training data)

mel_normalized = (mel_db - GLOBAL_MEAN) / GLOBAL_STD

# Method 3 is most robust — prevents test/train distribution mismatch

# Chapter 5: The Convolutional Neural Network (CNN)

## 5.1  Why a CNN?

CNNs are designed for grid-structured data — images. A mel spectrogram is, effectively, an image where:

- Width = time

- Height = frequency

- Pixel value = energy at that frequency and time

CNNs learn spatial patterns — clusters of bright pixels at specific locations. In spectrogram terms, that means the CNN learns: 'the A key has a bright burst at ~2000–4000 Hz immediately after impact, followed by a fade at ~500–1000 Hz.' These patterns distinguish keys from each other.

## 5.2  Building Blocks of the CNN

### Convolutional Layer (Conv2D)

The core building block. A convolutional layer slides small learnable filters across the spectrogram and detects local patterns.

from tensorflow.keras.layers import Conv2D

Conv2D(

filters=32,           # Number of different patterns to learn

kernel_size=(3, 3),   # Size of each filter (3×3 pixel patch)

activation='relu',    # Activation function

padding='same',       # Keep output same size as input

input_shape=(128, 5, 1)  # (height, width, channels)

)

| Parameter | Typical Value | Range | What It Controls |
| --- | --- | --- | --- |
| filters | 32, 64, 128 | 8 – 512 | Number of unique patterns learned. More filters = more capacity to learn complex patterns. |
| kernel_size | (3,3) | (2,2)–(7,7) | Size of the sliding detection window. 3×3 looks at a 3-frequency × 3-timeframe patch. |
| activation | 'relu' | Various | Non-linearity. ReLU (Rectified Linear Unit) sets negatives to zero. Required to learn complex patterns. |
| padding | 'same' | 'valid','same' | 'same' adds zeros around edges to keep spatial dimensions. 'valid' reduces size after each layer. |
| strides | (1,1) | (1,1)–(2,2) | How far the filter moves each step. (2,2) halves the spatial dimensions. |

### Batch Normalization

Normalizes the output of each layer to have zero mean and unit variance. This dramatically speeds up training and makes optimization more stable.

from tensorflow.keras.layers import BatchNormalization

# Applied after Conv2D, before activation:

model.add(Conv2D(32, (3,3), padding='same'))  # No activation here

model.add(BatchNormalization())               # Normalize

model.add(Activation('relu'))                 # Then activate

### MaxPooling2D

Reduces the spatial dimensions by taking the maximum value in each pooling window. This makes the network translation-invariant (a pattern slightly shifted in time/frequency still gets detected) and reduces computation.

from tensorflow.keras.layers import MaxPooling2D

MaxPooling2D(

pool_size=(2, 2),   # Look at 2×2 patches

strides=(2, 2),     # Move 2 steps each time (halves dimensions)

)

# Example: input (128, 10) → after MaxPool(2,2) → (64, 5)

### Dropout

During training, randomly sets a fraction of neuron outputs to zero. This prevents the network from becoming too reliant on any single neuron — a form of regularization that improves generalization.

from tensorflow.keras.layers import Dropout

Dropout(rate=0.25)  # 25% of neurons randomly zeroed each forward pass

Dropout(rate=0.5)   # 50% dropout — more aggressive regularization

# IMPORTANT: Dropout is active only during training.

# During inference/prediction, all neurons are active.

### Flatten

After the convolutional layers, the feature maps are 3D tensors. Flatten() reshapes them into a 1D vector so they can feed into dense layers.

from tensorflow.keras.layers import Flatten

# Example: (32, 2, 64 filters) → Flatten → (32 × 2 × 64,) = (4096,)

### Dense (Fully Connected) Layers

Traditional neural network layers where every neuron connects to every input. Used at the end of the CNN to combine all extracted features and produce the final classification.

from tensorflow.keras.layers import Dense

Dense(256, activation='relu')   # Hidden layer: 256 neurons

Dense(n_classes, activation='softmax')  # Output layer

# softmax converts raw scores → probabilities that sum to 1

# output[i] = probability that input belongs to class i

## 5.3  Complete Model Architecture

| Layer | Type / Config | Output Shape | Purpose |
| --- | --- | --- | --- |
| Input | (128, T, 1) | (128, T, 1) | Mel spectrogram. 128 mel bins × T time frames × 1 channel. |
| Conv2D(32) | 32 filters, 3×3 | (128, T, 32) | Detect low-level patterns (edges, onsets). |
| BatchNorm | - | (128, T, 32) | Stabilize training. |
| MaxPool(2,2) | pool 2×2 | (64, T/2, 32) | Reduce spatial size. |
| Conv2D(64) | 64 filters, 3×3 | (64, T/2, 64) | Detect mid-level patterns. |
| BatchNorm | - | (64, T/2, 64) | Stabilize. |
| MaxPool(2,2) | pool 2×2 | (32, T/4, 64) | Reduce size again. |
| Conv2D(128) | 128 filters, 3×3 | (32, T/4, 128) | High-level frequency-time patterns. |
| BatchNorm | - | (32, T/4, 128) | Stabilize. |
| GlobalAvgPool | - | (128,) | Collapse spatial dims. |
| Dropout(0.5) | - | (128,) | Regularization. |
| Dense(256) | relu | (256,) | Learn class-specific combinations. |
| Dropout(0.25) | - | (256,) | More regularization. |
| Dense(N_keys) | softmax | (N_keys,) | Output probability per key. |

## 5.4  Training Parameters

### Loss Function: Categorical Cross-Entropy

loss = 'categorical_crossentropy'

# Measures how wrong your predictions are.

# If true class is 'A' (one-hot: [1,0,0,0...])

# And prediction is [0.9, 0.05, 0.03, ...] → small loss (good)

# And prediction is [0.1, 0.4, 0.3, ...]  → large loss (bad)

# For binary (2 classes): use 'binary_crossentropy'

# For sparse integer labels: use 'sparse_categorical_crossentropy'

### Optimizer: Adam

from tensorflow.keras.optimizers import Adam

optimizer = Adam(

learning_rate=0.001,  # How large each gradient step is

beta_1=0.9,           # Momentum for first moment (gradient mean)

beta_2=0.999,         # Momentum for second moment (gradient variance)

epsilon=1e-7,         # Small number to prevent division by zero

)

# Adam is adaptive — it adjusts the learning rate per parameter.

# Generally the best default optimizer for audio/image tasks.

| Parameter | Typical Value | Range | What It Controls |
| --- | --- | --- | --- |
| learning_rate | 0.001 | 1e-5 – 0.01 | Step size for weight updates. Too high: training diverges. Too low: training is very slow. |
| batch_size | 32 | 8 – 256 | How many samples processed before weights update. Larger = more stable gradients, more memory. |
| epochs | 50–200 | 10 – 500 | How many times to loop through the full training dataset. |
| validation_split | 0.2 | 0.1 – 0.3 | Fraction of training data held out to monitor overfitting. |

## 5.5  Training Loop and Callbacks

from tensorflow.keras.callbacks import EarlyStopping, ReduceLROnPlateau

callbacks = [

# Stop training if validation loss doesn't improve for 10 epochs

EarlyStopping(monitor='val_loss', patience=10, restore_best_weights=True),

# Halve learning rate if stuck for 5 epochs

ReduceLROnPlateau(monitor='val_loss', factor=0.5, patience=5, min_lr=1e-6),

]

history = model.fit(

X_train, y_train,

batch_size=32,

epochs=100,

validation_split=0.2,

callbacks=callbacks,

)

# Chapter 6: Evaluation & Interpretation

## 6.1  Key Metrics

| Parameter | Typical Value | Range | What It Controls |
| --- | --- | --- | --- |
| Accuracy | — | 0–100% | Percentage of keystrokes correctly classified. Simple but misleading if classes are imbalanced. |
| Loss | — | 0 → ∞ | Cross-entropy loss. Lower = better. Watch for val_loss rising while train_loss falls (overfitting). |
| Precision | — | 0–1 | Of all times the model said 'key A', how often was it actually key A? |
| Recall | — | 0–1 | Of all actual key A presses, how many did the model correctly identify? |
| F1 Score | — | 0–1 | Harmonic mean of precision and recall. Best single metric when classes are balanced. |
| Confusion Matrix | — | N×N grid | Shows which keys get confused with which. Reveals if 'S' is often mistaken for 'A' etc. |

## 6.2  Interpreting the Confusion Matrix

from sklearn.metrics import confusion_matrix, classification_report

import seaborn as sns

y_pred = np.argmax(model.predict(X_test), axis=1)

y_true = np.argmax(y_test, axis=1)

cm = confusion_matrix(y_true, y_pred)

# Visualize

sns.heatmap(cm, annot=True, fmt='d', cmap='Blues',

xticklabels=key_labels, yticklabels=key_labels)

# High off-diagonal values = those keys are acoustically similar

# e.g., 'F' and 'G' might be confused — they're adjacent keys

## 6.3  Common Problems and How to Fix Them

🔴 Problem: Overfitting (val_loss rises while train_loss falls) Add more Dropout. Add L2 regularization. Collect more training data. Use data augmentation.

🔴 Problem: Underfitting (both losses plateau at high values) Increase model capacity (more filters, more layers). Reduce Dropout. Train for more epochs. Reduce learning rate.

🔴 Problem: All predictions are the same class Check class imbalance. Use class_weight parameter in model.fit(). Check that labels are correctly one-hot encoded.

## 6.4  Data Augmentation for Audio

If you have limited training data, augmentation helps by creating variations:

def augment_audio(audio, sr):

augmented = []

# Add Gaussian noise (simulates different microphones)

noise = audio + 0.005 * np.random.randn(len(audio))

augmented.append(noise)

# Time shift (slightly early or late detection)

shift = int(0.01 * sr)  # 10ms shift

shifted = np.roll(audio, shift)

augmented.append(shifted)

# Pitch shift (simulates different keyboard mechanisms)

pitched = librosa.effects.pitch_shift(audio, sr=sr, n_steps=1)

augmented.append(pitched)

return augmented

# Chapter 7: Master Parameter Reference

## 7.1  Audio Loading Parameters

| Parameter | Typical Value | Range | What It Controls |
| --- | --- | --- | --- |
| sr | 22050 | 8000–44100 | Sample rate. Match to your recording equipment or let librosa resample. |
| mono | True | True/False | Convert to single audio channel. Essential for consistent input. |
| offset | 0.0 | 0 → duration | Start loading from this time (seconds). Useful for trimming recordings. |
| duration | None | float | Only load this many seconds of audio. |

## 7.2  Mel Spectrogram Parameters

| Parameter | Typical Value | Range | What It Controls |
| --- | --- | --- | --- |
| n_mels | 128 | 40–256 | Mel frequency bins (height of spectrogram image). |
| n_fft | 2048 | 512–8192 | FFT window size. Controls frequency resolution. |
| hop_length | 512 | 128–1024 | Frame advance. Controls time resolution (width of spectrogram). |
| fmin | 0 | 0–500 | Lowest frequency to include in mel filterbank. |
| fmax | sr/2 | 1000–sr/2 | Highest frequency to include. |
| win_length | n_fft | < n_fft | Analysis window length. Usually equals n_fft. |
| window | 'hann' | Various | Windowing function. Hann is standard. |
| center | True | True/False | Pad signal so frame i is centered at sample i*hop_length. |

## 7.3  CNN Hyperparameters

| Parameter | Typical Value | Range | What It Controls |
| --- | --- | --- | --- |
| filters | 32/64/128 | 8–512 | Number of learned feature detectors per conv layer. |
| kernel_size | (3,3) | (2,2)–(7,7) | Filter spatial dimensions. |
| pool_size | (2,2) | (2,2)–(4,4) | MaxPooling reduction factor. |
| dropout_rate | 0.25–0.5 | 0.0–0.8 | Fraction of neurons randomly disabled during training. |
| learning_rate | 0.001 | 1e-5 – 0.1 | Optimizer step size. |
| batch_size | 32 | 8–256 | Samples per gradient update. |
| epochs | 100 | 10–500 | Training iterations over full dataset. |

## 7.4  Quick Tuning Guide

If accuracy is low: Increase n_mels (128 to 256). Try smaller n_fft (2048 to 1024) for better time resolution. Add more Conv layers. Increase filters.

If training is too slow: Reduce n_mels (128 to 64). Increase hop_length (512 to 1024). Reduce batch_size. Use GPU.

If model overfits: Add Dropout (0.25 to 0.5). Add L2 regularization. Add more training data. Increase n_fft for coarser features.

# Chapter 8: Complete Reference Implementation

Here is a self-contained reference implementation that ties together every concept in this guide:

import numpy as np

import librosa

import tensorflow as tf

from tensorflow.keras import layers, models

from sklearn.model_selection import train_test_split

from sklearn.preprocessing import LabelEncoder

# ─────────────────────────────────────────────

# CONFIGURATION

# ─────────────────────────────────────────────

SR = 22050          # Sample rate

N_MELS = 128        # Mel bins (spectrogram height)

N_FFT = 2048        # FFT window size

HOP_LENGTH = 512    # Hop length (time resolution)

FMIN = 0            # Min frequency

FMAX = SR // 2      # Max frequency (Nyquist)

DURATION = 0.1      # seconds per keystroke clip

TARGET_SAMPLES = int(DURATION * SR)  # 2205

# ─────────────────────────────────────────────

# FEATURE EXTRACTION

# ─────────────────────────────────────────────

def extract_mel(path):

y, _ = librosa.load(path, sr=SR, mono=True)

# Pad or truncate

if len(y) < TARGET_SAMPLES:

y = np.pad(y, (0, TARGET_SAMPLES - len(y)))

else:

y = y[:TARGET_SAMPLES]

# Mel spectrogram

mel = librosa.feature.melspectrogram(

y=y, sr=SR, n_mels=N_MELS,

n_fft=N_FFT, hop_length=HOP_LENGTH,

fmin=FMIN, fmax=FMAX

)

# Log compression

mel_db = librosa.power_to_db(mel, ref=np.max)

# Add channel dim → (128, T, 1)

return mel_db[..., np.newaxis]

# ─────────────────────────────────────────────

# MODEL DEFINITION

# ─────────────────────────────────────────────

def build_model(input_shape, n_classes):

model = models.Sequential([

layers.Input(shape=input_shape),

# Block 1

layers.Conv2D(32, (3,3), padding='same', activation='relu'),

layers.BatchNormalization(),

layers.MaxPooling2D((2,2)),

layers.Dropout(0.25),

# Block 2

layers.Conv2D(64, (3,3), padding='same', activation='relu'),

layers.BatchNormalization(),

layers.MaxPooling2D((2,2)),

layers.Dropout(0.25),

# Block 3

layers.Conv2D(128, (3,3), padding='same', activation='relu'),

layers.BatchNormalization(),

layers.GlobalAveragePooling2D(),

# Classifier

layers.Dense(256, activation='relu'),

layers.Dropout(0.5),

layers.Dense(n_classes, activation='softmax'),

])

model.compile(

optimizer=tf.keras.optimizers.Adam(learning_rate=0.001),

loss='categorical_crossentropy',

metrics=['accuracy']

)

return model

# ─────────────────────────────────────────────

# TRAINING

# ─────────────────────────────────────────────

# Load and encode labels

# X = np.array([extract_mel(p) for p in audio_files])

# y = tf.keras.utils.to_categorical(encoded_labels)

# X_train, X_test, y_train, y_test = train_test_split(X, y, test_size=0.2)

# model = build_model(input_shape=X_train.shape[1:], n_classes=n_classes)

# history = model.fit(X_train, y_train, batch_size=32, epochs=100,

#                     validation_data=(X_test, y_test))

# Glossary of Terms

Amplitude: The magnitude (loudness) of a sound wave at a given moment. Stored as a float between -1.0 and 1.0 in normalized audio.

Batch Normalization: Normalizes layer activations to have zero mean and unit variance. Speeds up training and improves stability.

CNN (Convolutional Neural Network): A neural network architecture designed for grid data (images). Uses convolutional filters to detect local patterns.

Convolution: Mathematical operation where a small filter slides across the input, computing dot products to detect patterns.

dB (Decibel): Logarithmic unit for measuring signal intensity. 0 dB = reference level; -80 dB = very quiet.

Dropout: Regularization technique that randomly zeros out neurons during training to prevent overfitting.

FFT (Fast Fourier Transform): Efficient algorithm to decompose a signal into its component frequencies.

Filterbank: A set of bandpass filters that divide the frequency spectrum. Mel filterbanks are spaced on the Mel scale.

Hop Length: The number of samples between successive STFT analysis frames. Controls time resolution.

Librosa: Python library for audio analysis. Used for loading audio files and computing spectrograms.

MaxPooling: Downsampling operation that takes the maximum value in each pooling window.

Mel Scale: A perceptual frequency scale that approximates how the human auditory system perceives pitch.

Mel Spectrogram: A spectrogram where frequency bins are spaced according to the Mel scale.

n_fft: FFT window size. Controls frequency resolution — larger = finer frequency detail.

n_mels: Number of Mel frequency bins. Determines the height of the spectrogram image.

Nyquist Frequency: Maximum detectable frequency = sr/2. Above this, aliasing occurs.

Overfitting: Model memorizes training data but fails to generalize. Validation loss rises while training loss falls.

ReLU: Rectified Linear Unit. Activation function: f(x) = max(0, x). Introduces non-linearity.

Sample Rate (sr): Number of audio samples recorded per second. Determines maximum frequency content.

Side-Channel Attack: Attack that exploits information leaked through physical processes (sound, power, EM radiation) rather than direct access.

Softmax: Converts raw output scores to probabilities that sum to 1. Used in the output layer for classification.

Spectrogram: 2D representation of audio showing how frequency content changes over time.

STFT (Short-Time Fourier Transform): Applies FFT to short overlapping windows of audio. Produces a time-frequency representation.

Document prepared for educational use · Based on ayushma18/keyboard_sound